HEATHKIT DX-100 AUDIO MOD

Audio Mod

1) Replace cathode caps from pin 8 of the 12AX7 to gnd with a 220uf 35v electrolytic and 1uf 35v tantalum. 2) Replace 510pf coupling cap between pin 6 of the 12AX7 and the volume control with a .05uf 600v metalized polyester film. 3) Remove the 2uf cap from pin 3 of the 12AX7 to gnd. Pin 3 will be the point where negative feedback is injected. See Figure 1 4) Replace the .1uf supply decoupling caps with 10uf 450v electrolytic caps. (one is on the 47K dropping resistor and the other is on the 100K) 5) Replace the 510pf coupling cap between pin 1 of the 12AX7 and pin 2 of the 12BY7 with a .05uf 600v metalized polyester film. 6) Replace the 2uf cathode cap on pin 1of the 12BY7 to gnd. with two caps. One 100uf 35v electrolytic and one 1uf 35v tantalum. 7) On the power supply side of the driver transformer primary, (labeled "M") add two .05uf 500v ceramic disc caps to gnd. 8) On the secondary center-tap of the driver transformer, add a 1000uf 100v electrolytic cap to gnd. (+ side to gnd.) 9) On the mod transformer where the primary center-tap and secondary hot side pick up +800v, add a .01uf 3kv ceramic disc cap to gnd. 10) On the mod transformer, replace the .02uf 1.5kv cap across the secondary with a .01uf 3kv ceramic disc. 11) Replace the .1uf decoupling cap on pin 3 of the 1625s to gnd with a .1uf 600v metalized polyester film. 12) From the 500ohm tap on the mod transformer to pin 3 of the 12AX7, build the circuit shown in figure-1 to apply the negative feedback.